rPPG Signal Estimation for Drunkenness Detection Using RGB Videos: A Literature Review

- INTRODUCTION

Drunk driving is still a major road safety problem around the world, and it leads to thousands of accidents every year that could have been avoided. For a long time the main way to check if someone is intoxicated has been the breathalyser, but this method is invasive because the driver has to physically blow into the device. Remote photoplethysmography (rPPG) is a possible alternative that does not need any contact at all. It works by using a normal RGB camera to pick up the small physiological changes that happen when a person has been drinking.

- METHODOLOGY FOR PAPER SELECTION

I searched for papers using a few academic databases, mainly IEEE Xplore and Google Scholar. The keywords I used were "remote photoplethysmography", "rPPG", "drunk detection", "intoxication detection", "driver monitoring" and "facial video analysis". I mostly looked at papers published between 2018 and 2026, since this is where most of the recent deep learning work has come out.

I only kept a paper if it met a few conditions: it had to be about rPPG signal extraction or intoxication detection, it had to use deep learning or another AI method and it had to say something about the datasets and metrics used. In total I chose 10 papers, and together they cover different parts of this area: the basic rPPG techniques [5][7], the newer state-of-the-art architectures [2][9], a benchmark toolbox [6], papers focused on driving situations [3][4], and systems built specifically for intoxication detection [1][8][10].

- KEY FINDINGS

A. rPPG Fundamentals and Traditional Methods

rPPG measures the blood volume pulse by looking at the very small colour changes in a person’s skin, and it does this with a normal camera [7]. The older methods used techniques like Independent Component Analysis (ICA) and chrominance-based methods such as CHROM to pull the pulse signal out of the video [5][7]. The main issue with these methods is that they do not cope well with movement or changes in lighting, which are very common in real situations.

B. Deep Learning Architectures

More recent rPPG work has moved towards deep learning that runs from start to finish. Some of the convolutional models include DeepPhys (a two-branch 2D CNN with attention) and PhysNet (a 3D CNN), which uses temporal shift operations to make it more efficient [6]. Transformer models like PhysFormer can pick up patterns over longer time periods, and they usually work better when tested on data from different sources [6]. Spiking-PhysFormer lowers energy use by 10.1% and still reaches a 2.80 bpm Mean Absolute Error (MAE) (using neuromorphic), which is useful for devices that run on a battery [9].

PhysVLM tries to deal with the problem of working across different domains by using a vision-language model (VLM) to get physiological features like heart rate. Doing this brings MAE down from 5.85 to 2.85 bpm on some of the harder datasets [2]. The reason it works well is that it pays attention to the periodic features of the signal instead of visual things like skin tone.

- COMPARATIVE ANALYSIS

| Paper | Method | Dataset | Key Results | Metrics | Strengths | Limitations |
| --- | --- | --- | --- | --- | --- | --- |
| [1] | Ensemble ML (GB, RF) + DL (GoogleNet, Xception) | Custom drunk/sober images with noise augmentation | 99.8% accuracy (low noise) | Accuracy | XAI integration, FPGA implementation | Lab conditions only |
| [2] | PhysVLM (Vision-Language + Transformer) | NBHR, clinical preterm infants | MAE 5.85→2.85 bpm | MAE, cross-domain | Robust generalization, physiology-guided | Limited to HR/RR |
| [3] | IAT + Yolov5face + RhythmNet | MR-NIRP Car dataset | 8.89 bpm RMSE | RMSE | Real driving conditions | Small motion only |
| [4] | Systematic review (29 studies) | Multiple driver monitoring datasets | N/A (review) | N/A | Comprehensive analysis | No new method |
| [5] | DL review (CNN, Transformer, GAN) | MAHNOB-HCI, PURE, VIPL-HR | Varies by method | MAE, RMSE, Pearson | Broad coverage | No implementation |
| [6] | 6 unsupervised + 6 supervised models | 6 datasets (UBFC, PURE, MMPD, etc.) | PhysNet: 0.98 bpm MAE | MAE, RMSE, SNR, ρ | Open-source, reproducible | HR only, no HRV/BP |
| [7] | Systematic review (376 papers) | Multiple HR/SpO2 datasets | N/A (review) | Various | Clinical focus, bias discussion | Identifies gaps only |
| [8] | MobileNet CNN on thermal IR | 50 Thai participants, 4 classes | 85.10% (4-class), 74.07% (binary) | Accuracy | Contactless, works in dark | High false positive, small dataset |
| [9] | Spiking-PhysFormer (HNN with SNN) | PURE, UBFC-rPPG, UBFC-Phys, MMPD | 2.80 bpm MAE, 1.34 mJ/frame | MAE, MAPE, energy | 10.1% energy reduction | Not tested on real neuromorphic chips |
| [10] | LSTM on facial features (EAR, gaze, head movement) | 60 participants, simulator, 3 BAC levels | 75% accuracy, 85% sober detection | Accuracy | Immediate prevention, inexpensive | 21% mild/severe confusion |

- RESEARCH GAP IDENTIFIED

Even though a lot of progress has been made in both rPPG and intoxication detection, there is still one clear gap in the research: no existing work combines state-of-the-art transformer-based rPPG signal extraction with drunk/sober classification optimized for real-time in-vehicle deployment. Most current drunk detection systems use facial features or thermal imaging instead of rPPG signals. However, heart rate and heart rate variability are the physiological signals that alcohol actually affects body, making rPPG a more direct indicator of intoxication than appearance-based methods. Current systems focusing on facial landmarks or temperature changes miss this direct physiological link. Although the DIF (Drunk in the Wild) dataset provides real-world videos from YouTube, Periscope, and Twitch, it remains largely underutilized in recent research. A systematic approach using this real-world data could significantly improve generalization to actual driving scenarios.

While energy-efficient architectures for rPPG already exist, no work has properly analyzed the trade-offs between accuracy, latency, and computational cost specifically for intoxication detection in vehicles. Understanding these trade-offs is essential for practical deployment in resource-constrained automotive systems.

- PROPOSED APPROACH / METHODOLOGY

To address these gaps, this project implements a two-stage pipeline designed for practical in-vehicle deployment.

The first stage focuses on rPPG signal extraction. The project implements a model, a state-of-the-art transformer-based architecture specifically designed for hierarchical temporal periodic feature learning from video. This model processes facial videos to automatically extract physiological features including heart rate, heart rate variability, frequency domain characteristics, and pulse morphology. The preprocessing pipeline begins with face detection using YOLO V5, followed by region of interest (ROI) extraction from the forehead and cheeks where pulse signals are strongest. Frames are then normalized to enhance signal quality.

The second stage is drunk/sober classification. Multiple classifiers are trained on the physiological features extracted in Stage 1. An Support Vector Machine (SVM) classifier serves as the baseline, while a Random Forest classifier helps identify which physiological features are most important for classification. A Long Shirt Term Memory (LSTM) network is also trained to capture temporal patterns in the rPPG signals. The system evaluates both binary classification (drunk versus sober) and multi-class classification (sober, mildly intoxicated, and severely intoxicated) depending on data availability.

The third stage addresses real-time optimization, which is critical for in-vehicle deployment. The project applies model compression techniques including pruning to remove less important network weights. The system then systematically evaluates the trade-offs between accuracy, latency, and computational cost. The deployment target is an inference latency below 100 milliseconds per frame while maintaining classification accuracy above 85 percent, which would make the system practical for real-time monitoring in vehicles.

- DATASETS

Primary Dataset: the main dataset will be DIF (Drunk in the Wild). It gives links to YouTube, Periscope and Twitch videos of people who are drunk and sober. I will download the videos from these links, and the final size of the dataset will depend on how many links still work.

- PROJECT PLAN (Per week May 10th to August 20th )

- USE OF AI

AI was used in a small way to help improve the writing and to fix grammar and spelling mistakes. It was also used to summarize long research papers. The research and the ideas in this review are my own.

- REFERENCES

[1] Alsulieman, R., Abbass, M., Swilley, R., Sherif, A., Elsersy, M., Abdelfattah, R. and Khalil, K., 2026. Identification of Drinking Intoxication: Applying Artificial Intelligence for Improved Traffic Safety. IEEE Internet of Things Journal.

[2] Gao, J., San, I.W., Luo, X., Chen, Z., Yu, Z., Tan, T. and Sun, Y., 2026. PhysVLM: Vision-language Model for Generalizable Multi-task Remote Physiological Measurement. IEEE Transactions on Instrumentation and Measurement.

[3] Huang, J., Zai, W., He, Y. and Jin, Z., 2025, May. A Remote End-to-end Real-time Heart Rate Detection Method Based on rPPG in Driving Scenarios. In 2025 8th International Conference on Artificial Intelligence and Big Data (ICAIBD) (pp. 225-237). IEEE.

[4] Ahmed, S.G., Verbert, K., Zaki, N., Khalil, A., Aljassmi, H. and Alnajjar, F., 2025. AI innovations in rPPG systems for driver monitoring: Comprehensive systematic review and future prospects. Ieee Access, 13, pp.22893-22918.

[5] Debnath, U. and Kim, S., 2025. A comprehensive review of heart rate measurement using remote photoplethysmography and deep learning. BioMedical Engineering OnLine, 24(1), p.73.

[6] Liu, X., Narayanswamy, G., Paruchuri, A., Zhang, X., Tang, J., Zhang, Y., Sengupta, R., Patel, S., Wang, Y. and McDuff, D., 2023. rppg-toolbox: Deep remote ppg toolbox. Advances in Neural Information Processing Systems, 36, pp.68485-68510.

[7] Pirzada, P., Wilde, A. and Harris-Birtill, D., 2024. Remote photoplethysmography for heart rate and blood oxygenation measurement: A review. IEEE Sensors Journal, 24(15), pp.23436-23453.

[8] Thorupunoori, S., Billakanti, K., Hemanth, T.C., Sungomula, S.R., Gurram, S., Kaur, H. and Rakhra, M., 2021, September. Camera based drunks detection mechanism integrated with DL (deep learning). In 2021 9th International Conference on Reliability, Infocom Technologies and Optimization (Trends and Future Directions)(ICRITO) (pp. 1-4). IEEE.

[9] Liu, M., Tang, J., Chen, Y., Li, H., Qi, J., Li, S., Wang, K., Gan, J., Wang, Y. and Chen, H., 2025. Spiking-PhysFormer: Camera-based remote photoplethysmography with parallel spike-driven transformer. Neural Networks, 185, p.107128.

[10] Keshtkaran, E., Von Berg, B., Regan, G., Suter, D. and Gilani, S.Z., 2024. Estimating blood alcohol level through facial features for driver impairment assessment. In Proceedings of the IEEE/CVF Winter Conference on Applications of Computer Vision (pp. 4539-4548).